NETWORK NOTES (Crash Course)

Early computer networks (1950s–60s)

- Goal: share information/ideas between computers

- Before networks: data moved physically

  - punch cards

  - reels of magnetic tape

  - “sneakernet” (humans carrying data)

LAN (Local Area Network)

Ethernet (Bob Metcalfe, 1973)

- A LAN is a network in a small geographic area (building, office, etc.)

- Early Ethernet: multiple computers on a single shared cable

- When a computer sends data:

  - it writes electrical signals onto the cable

  - the signal is broadcast to all computers

Why broadcast isn’t chaos

Each device has a MAC address (Media Access Control)

- MAC is like a “hardware ID”

- Ethernet frames include a header

  - contains destination MAC address

- Computers only process frames where the destination MAC matches theirs

📌 Key vocab:

- Ethernet “packet” is more precisely called a frame

- “header” holds addressing + metadata

CSMA (Carrier Sense Multiple Access)

Carrier = shared transmission medium carrying data

Examples:

- copper wire (Ethernet)

- air / radio waves (Wi-Fi)

Bandwidth = rate a carrier can transmit data
 (more bandwidth = more data per second)

Shared carrier drawback

When traffic increases, two devices can transmit at the same time.

Collisions

Collision = two transmissions overlap → data becomes garbled/unusable

What happens after collision:

- devices detect collision

- then they retry sending later after waiting a random time

Exponential Backoff
If collisions continue, devices wait longer each time:

1 → 2 → 4 → 8 → 16 seconds (etc.)

- reduces repeat collisions by spreading retries out

Collision Domain

- Collision domain = group of devices on a shared medium where collisions can occur

- Performance improves when you shrink collision domains

  - fewer devices share the same carrier → fewer collisions

Example:

- 6 computers on one shared cable = one collision domain

Switches (solving collisions)

A switch splits a network into smaller collision domains.

Example:
A B C | SWITCH | D E F

- Switch keeps track of what MAC addresses are on which side/port (MAC address table)

- If A → C (same side):

  - switch does not forward to the other side

  - devices on D/E/F side can transmit independently

- If F → A:

  - switch forwards across → both sides used (briefly)

✅ This is how modern LANs are built.

📌 Micro-upgrade: Modern Ethernet with switches is usually full-duplex (send & receive simultaneously) and collisions basically disappear. Collisions are mainly an “old shared cable/hub” era thing.

Routing & Switching Methods

Dedicated lines (early networks)

- simplest connection: allocate a dedicated communication line between endpoints

- similar to early telephone networks

Circuit Switching

- route traffic by reserving an entire circuit end-to-end

- great for voice calls

- inefficient for bursty data

Message Switching

- messages move hop-to-hop (store-and-forward)

- more reliable/fault tolerant than circuit switching

- but messages can be huge and clog the network

Hop Count

- number of hops along a route

- can help identify routing issues (ex: loops)

📌 Hop limit / TTL

- prevents packets from bouncing forever

- each router decrements TTL; when it hits 0, packet gets dropped

Packet Switching (foundation of the Internet)

- big messages get chopped into packets

- packets travel flexible routes using available capacity

- efficient + fault tolerant → this is the internet

Internet Protocol (IP)

- defines the format for addressing + routing packets (created in the 1970s)

- like physical mail: packets need a valid destination address

- IP has size limits (like weight/size limits on mail)

Congestion Control

Routers + networks try to prevent overload by:

- balancing traffic across routes

- preventing bottlenecks

Internet Coordination Protocols

ARPANET

- first major packet-switched network → ancestor of modern internet

Backbone

- huge routers + high bandwidth links connecting regions

Traceroute

- shows path (hops) a packet takes

BGP + ICMP

- BGP = how large networks (ISPs) exchange routing information on the internet

- ICMP = “control + diagnostic messages” (e.g., ping, traceroute behavior)

UDP vs TCP (Transport Layer)

UDP (User Datagram Protocol)

- transport protocol inside IP packets

- adds a header including:

  - port number (which app/service)

  - checksum (data integrity check)

Ports

- OS gives programs unique ports

- when packet arrives:

  - OS reads port

  - routes data to correct program

✅ IP gets packet to the right computer
✅ UDP gets packet to the right program

UDP Checksum

- sender computes checksum over data

- receiver recomputes and compares

- if mismatch:

  - packet is corrupt → usually discarded

- UDP does not guarantee delivery or retransmit

📌 Micro-upgrade: UDP checksum isn’t always a simple “sum of bytes” in the real world — it’s a defined algorithm and includes extra header info (“pseudo-header”) — but your conceptual understanding is correct: detect corruption, don’t fix it.

TCP (Transmission Control Protocol)

- transport protocol inside IP packets (TCP/IP)

- guaranteed delivery

- packets have sequence numbers

- receiver sends ACKs (acknowledgements)

TCP handles:

- out-of-order delivery

- dropped packets (retransmission)

- congestion control / throttling based on conditions

DNS (Domain Name System)

- “phonebook for the internet”

- maps domain names → IP addresses

Domain structure:

- TLD: .com, .gov, .net

- second-level: google.com

- subdomain: drive.google.com

OSI Model (Networking Layers)

- Physical

- Data Link

- Network

- Transport

- Session

- Presentation

- Application

📌 Micro-upgrade: People often simplify dev-use of OSI into:

- L1–L2: hardware + Ethernet/Wi-Fi

- L3: IP

- L4: TCP/UDP

- L7: HTTP/APIs

That simplified map is the one you’ll use in debugging.

WWW Basics

- fundamental unit: web page

- a page is a document that can include hyperlinks to other pages

Hyperlinked information (origins)

- Value of hyperlinked info conceived by Vannevar Bush (1945)

- Described a hypothetical machine called Memex

- Key idea: associative indexing

  - tying two items together

  - later, viewing one can instantly recall the other

Hypertext = text containing hyperlinks
Web pages are the most common hypertext documents today, retrieved and rendered by browsers.

URLs + Getting a webpage

URL (Uniform Resource Locator)

- each web page needs a unique address so pages can link to each other

What happens when you request a site

- DNS lookup

  - domain → IP address

- browser opens TCP connection to the web server

- browser requests hypertext via HTTP (Hypertext Transfer Protocol)

- server sends response

- browser renders it

Early HTTP

- first web standard: HTTP 0.9 (1991)

- only supported one method: GET

- request was plain text (ASCII)

- server responds with the requested page

Following a hyperlink → sends another GET request.

Status codes

HTTP responses include a status code, e.g.

- 404 = not found

📌 Upgrade: status codes are part of the HTTP response line and grouped like:

- 2xx = success

- 3xx = redirect

- 4xx = client error

- 5xx = server error

Why HTML was needed

Web pages were sent as plain text (ASCII / UTF-8 / UTF-16 etc.)

But plain text alone can’t express:

- links

- headings

- images

- layout

So we needed markup.

HTML

- very early HTML appeared around 1990

- (your “18 commands” note is good — early HTML was tiny)

- HTML lets a page label elements like links, headings, paragraphs, etc.

Tim Berners-Lee: The Web Trinity

- Built the first web browser + web server in 1990 at CERN

- Created:
✅ URLs
✅ HTML
✅ HTTP

- Released the web publicly in 1991

Mosaic (huge milestone)

Mosaic browser

- embedded graphics alongside text

- user-friendly GUI

- bookmarks

This was the “web goes mainstream” turning point.

Early Search Engines

JumpStation (1993) by Jonathan Fletcher
Core components:

- web crawler (spider)

  - follows links, discovers pages

- index

  - records what words appear on what pages

- search algorithm

  - ranks results using the index

Early ranking exploit:

- keyword stuffing (spam the same word repeatedly)

Google’s breakthrough (conceptually)

Google used backlinks (links from other sites pointing to you) as a quality signal.

- more/lower quality spam became harder

- links became “votes”

📌 Upgrade: This idea is closely associated with PageRank (late 1990s), where links help infer authority.